8086 Microprocessor Online Compiler Lab Manual

Experiment 1: 8-Bit Addition

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AL, 0x12 | B0 12 | Load 8-bit hex value 12H into AL register |
| 0102 | MOV BL, 0x05 | B3 05 | Load 8-bit hex value 05H into BL register |
| 0104 | ADD AL, BL | 00 D8 | Add BL to AL, result accumulates in AL |
| 0106 | MOV CL, AL | 88 C1 | Copy final sum to CL for visual backup verification |
| 0108 | INT 0x03 | CC | Breakpoint Halt - safely stops execution |

Experiment 2: 8-Bit Subtraction

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AL, 0x12 | B0 12 | Load minuend 12H into AL register |
| 0102 | MOV BL, 0x05 | B3 05 | Load subtrahend 05H into BL register |
| 0104 | SUB AL, BL | 28 D8 | Subtract BL from AL, result stays in AL |
| 0106 | MOV CL, AL | 88 C1 | Copy final difference to CL register |
| 0108 | INT 0x03 | CC | Breakpoint Halt |

Experiment 3: 16-Bit Addition

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AX, 0x1234 | B8 34 12 | Load 16-bit value 1234H into AX register |
| 0103 | MOV BX, 0x0002 | BB 02 00 | Load 16-bit value 0002H into BX register |
| 0106 | ADD AX, BX | 01 D8 | Add BX to AX, sum saves in AX |
| 0108 | INT 0x03 | CC | Breakpoint Halt |

Experiment 4: 16-Bit Subtraction

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AX, 0x4567 | B8 67 45 | Load 16-bit value 4567H into AX register |
| 0103 | MOV BX, 0x1234 | BB 34 12 | Load 16-bit value 1234H into BX register |
| 0106 | SUB AX, BX | 29 D8 | Subtract BX from AX, result stays in AX |
| 0108 | INT 0x03 | CC | Breakpoint Halt |

Experiment 5: 8-Bit Multiplication (Native)

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AL, 0x05 | B0 05 | Load multiplicand 05H into AL register |
| 0102 | MOV BL, 0x03 | B3 03 | Load multiplier 03H into BL register |
| 0104 | MUL BL | F6 E3 | Multiply AL by BL. Result saves automatically in AX. |
| 0106 | INT 0x03 | CC | Breakpoint Halt |

Experiment 6: 8-Bit Division (Native)

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Mandatory execution entry point label |
| 0100 | MOV AX, 0x000B | B8 0B 00 | Load 16-bit dividend 000BH (11) into AX register |
| 0103 | MOV BL, 0x03 | B3 03 | Load 8-bit divisor 03H into BL register |
| 0105 | DIV BL | F6 F3 | Divide AX by BL. Quotient sits in AL, Remainder in AH. |
| 0107 | INT 0x03 | CC | Breakpoint Halt |