1. 8-Bit Addition

start:
MOV AL, 0x12
MOV BL, 0x05
ADD AL, BL
MOV CL, AL
INT 0x03

2. 8-Bit Subtraction

start:
MOV AL, 0x12
MOV BL, 0x05
SUB AL, BL
MOV CL, AL
INT 0x03

3. Factorial of a Number

start:
MOV CX, 0x0005
MOV AX, 0x0001
fact_loop:
MUL CX
LOOP fact_loop
MOV DX, AX
INT 0x03

4. Square of a Number

start:
MOV AL, 0x09
MOV BL, AL
MUL BL
MOV CX, AX
INT 0x03

5. Find Biggest of 5 Numbers

start:
MOV AL, 0x14
MOV BL, 0x32
CMP AL, BL
JAE check_3
MOV AL, BL
check_3:
MOV BL, 0x0A
CMP AL, BL
JAE check_4
MOV AL, BL
check_4:
MOV BL, 0x5F
CMP AL, BL
JAE check_5
MOV AL, BL
check_5:
MOV BL, 0x02
CMP AL, BL
JAE done_big
MOV AL, BL
done_big:
MOV DL, AL
INT 0x03

6. Find Smallest of 5 Numbers

start:
MOV AL, 0x14
MOV BL, 0x32
CMP AL, BL
JBE check3_s
MOV AL, BL
check3_s:
MOV BL, 0x0A
CMP AL, BL
JBE check4_s
MOV AL, BL
check4_s:
MOV BL, 0x5F
CMP AL, BL
JBE check5_s
MOV AL, BL
check5_s:
MOV BL, 0x02
CMP AL, BL
JBE done_small
MOV AL, BL
done_small:
MOV DL, AL
INT 0x03