8086 Web Simulator Complete Lab Manual

1. 8-Bit Addition

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV AL, 0x12 | B0 12 | Load 1st number into AL |
| 0102 | MOV BL, 0x05 | B3 05 | Load 2nd number into BL |
| 0104 | ADD AL, BL | 00 D8 | AL = AL + BL |
| 0106 | MOV CL, AL | 88 C1 | Copy answer to CL for visualization |
| 0108 | INT 0x03 | CC | Halt |

2. 8-Bit Subtraction

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV AL, 0x12 | B0 12 | Load minuend into AL |
| 0102 | MOV BL, 0x05 | B3 05 | Load subtrahend into BL |
| 0104 | SUB AL, BL | 28 D8 | AL = AL - BL |
| 0106 | MOV CL, AL | 88 C1 | Copy difference to CL |
| 0108 | INT 0x03 | CC | Halt |

3. Factorial of a Number

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV CX, 0x0005 | B9 05 00 | Set number to find factorial of (e.g., 5) |
| 0103 | MOV AX, 0x0001 | B8 01 00 | Initialize product accumulator to 1 |
| 0106 | fact_loop: |  | Loop label |
| 0106 | MUL CX | F7 E1 | AX = AX * CX |
| 0108 | LOOP fact_loop | E2 FC | Decrement CX, loop if CX != 0 |
| 010A | MOV DX, AX | 89 C2 | Copy final factorial value to DX |
| 010C | INT 0x03 | CC | Halt |

4. Square of a Number

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV AL, 0x09 | B0 09 | Load number to square into AL (e.g., 9) |
| 0102 | MOV BL, AL | 88 C3 | Copy the number to BL |
| 0104 | MUL BL | F6 E3 | Multiply AL by BL (AL = AL * BL) |
| 0106 | MOV CX, AX | 89 C1 | Copy 16-bit squared result into CX |
| 0108 | INT 0x03 | CC | Halt |

5. Find Biggest of 5 Numbers

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV AL, 0x14 | B0 14 | Initialize Max with 1st element (e.g., 20) |
| 0102 | MOV BL, 0x32 | B3 32 | Load 2nd element (e.g., 50) |
| 0104 | CMP AL, BL | 3A C3 | Compare Max with 2nd element |
| 0106 | JAE check_3 | 73 02 | If Max >= BL, skip update |
| 0108 | MOV AL, BL | 8A C3 | Update Max with BL value |
| 010A | check_3: |  | Label |
| 010A | MOV BL, 0x0A | B3 0A | Load 3rd element (e.g., 10) |
| 010C | CMP AL, BL | 3A C3 | Compare |
| 010E | JAE check_4 | 73 02 | Skip if Max >= BL |
| 0110 | MOV AL, BL | 8A C3 | Update Max |
| 0112 | check_4: |  | Label |
| 0112 | MOV BL, 0x5F | B3 5F | Load 4th element (e.g., 95) |
| 0114 | CMP AL, BL | 3A C3 | Compare |
| 0116 | JAE check_5 | 73 02 | Skip if Max >= BL |
| 0118 | MOV AL, BL | 8A C3 | Update Max |
| 011A | check_5: |  | Label |
| 011A | MOV BL, 0x02 | B3 02 | Load 5th element (e.g., 2) |
| 011C | CMP AL, BL | 3A C3 | Compare |
| 011E | JAE done_big | 73 02 | Skip if Max >= BL |
| 0120 | MOV AL, BL | 8A C3 | Update Max |
| 0122 | done_big: |  | Final label |
| 0122 | MOV DL, AL | 88 C2 | Copy absolute Biggest value to DL register |
| 0124 | INT 0x03 | CC | Halt |

6. Find Smallest of 5 Numbers

| Offset Address | Instruction | Machine Code (Hex) | Comments |
| --- | --- | --- | --- |
| 0100 | start: |  | Entry point |
| 0100 | MOV AL, 0x14 | B0 14 | Initialize Min with 1st element (e.g., 20) |
| 0102 | MOV BL, 0x32 | B3 32 | Load 2nd element (e.g., 50) |
| 0104 | CMP AL, BL | 3A C3 | Compare Min with 2nd element |
| 0106 | JBE check3_s | 76 02 | If Min <= BL, skip update |
| 0108 | MOV AL, BL | 8A C3 | Update Min with BL value |
| 010A | check3_s: |  | Label |
| 010A | MOV BL, 0x0A | B3 0A | Load 3rd element (e.g., 10) |
| 010C | CMP AL, BL | 3A C3 | Compare |
| 010E | JBE check4_s | 76 02 | Skip if Min <= BL |
| 0110 | MOV AL, BL | 8A C3 | Update Min |
| 0112 | check4_s: |  | Label |
| 0112 | MOV BL, 0x5F | B3 5F | Load 4th element (e.g., 95) |
| 0114 | CMP AL, BL | 3A C3 | Compare |
| 0116 | JBE check5_s | 76 02 | Skip if Min <= BL |
| 0118 | MOV AL, BL | 8A C3 | Update Min |
| 011A | check5_s: |  | Label |
| 011A | MOV BL, 0x02 | B3 02 | Load 5th element (e.g., 2) |
| 011C | CMP AL, BL | 3A C3 | Compare |
| 011E | JBE done_small | 76 02 | Skip if Min <= BL |
| 0120 | MOV AL, BL | 8A C3 | Update Min |
| 0122 | done_small: |  | Final label |
| 0122 | MOV DL, AL | 88 C2 | Copy absolute Smallest value to DL register |
| 0124 | INT 0x03 | CC | Halt |