8085 Assembly Code - Pure Text Copy/Paste Edition

Clean text blocks containing only assembly commands for quick pasting into your simulator/assembler editor.

1. 8-Bit Addition

MVI A, 12H
MVI B, 05H
ADD B
STA 8500H
HLT

2. 8-Bit Subtraction

MVI A, 12H
MVI B, 05H
SUB B
STA 8500H
HLT

3. 16-Bit Addition

LXI H, 1234H
LXI D, 0002H
DAD D
SHLD 8500H
HLT

4. 16-Bit Subtraction

LXI H, 4567H
LXI B, 1234H
MOV A, L
SUB C
MOV L, A
MOV A, H
SBB B
MOV H, A
SHLD 8500H
HLT

5. 8-Bit Multiplication

MVI A, 05H
MOV E, A
MVI B, 03H
MVI A, 00H
MVI C, 00H
LOOP: ADD E
JNC NEXT
INR C
NEXT: DCR B
JNZ LOOP
STA 8500H
MOV A, C
STA 8501H
HLT

6. 8-Bit Division

MVI A, 0BH
MVI B, 03H
MVI C, 00H
LOOP: CMP B
JC END
SUB B
INR C
JMP LOOP
END: STA 8501H
MOV A, C
STA 8500H
HLT

7. Addition of 10 Numbers

LXI H, 8200H
MVI B, 0AH
MVI A, 00H
MVI C, 00H
LOOP: ADD M
JNC NEXT
INR C
NEXT: INX H
DCR B
JNZ LOOP
STA 8500H
MOV A, C
STA 8501H
HLT

8. Find Biggest Number

LXI H, 8200H
MVI B, 05H
MOV A, M
DCR B
LOOP: INX H
CMP M
JNC NEXT
MOV A, M
NEXT: DCR B
JNZ LOOP
STA 8500H
HLT

9. Find Smallest Number

LXI H, 8200H
MVI B, 05H
MOV A, M
DCR B
LOOP: INX H
CMP M
JC NEXT
MOV A, M
NEXT: DCR B
JNZ LOOP
STA 8500H
HLT