8085 Microprocessor Lab Opcodes & Program Reference

A compilation of exact hex lookup codes and execution tables for required lab experiments.

Experiment 1: 8-Bit Addition

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | MVI A, 12H | 3E | Load 1st 8-bit number into Accumulator |
| 8001H |  | 12 | Immediate data (1st number) |
| 8002H | MVI B, 05H | 06 | Load 2nd 8-bit number into Register B |
| 8003H |  | 05 | Immediate data (2nd number) |
| 8004H | ADD B | 80 | Add Register B to Accumulator (A = A + B) |
| 8005H | STA 8500H | 32 | Store result direct to memory location 8500H |
| 8006H |  | 00 | Lower byte of target address |
| 8007H |  | 85 | Higher byte of target address |
| 8008H | HLT | 76 | Halt execution |

Experiment 2: 8-Bit Subtraction

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | MVI A, 12H | 3E | Load minuend into Accumulator |
| 8001H |  | 12 | Immediate data (minuend) |
| 8002H | MVI B, 05H | 06 | Load subtrahend into Register B |
| 8003H |  | 05 | Immediate data (subtrahend) |
| 8004H | SUB B | 90 | Subtract B from Accumulator (A = A - B) |
| 8005H | STA 8500H | 32 | Store result direct to memory location 8500H |
| 8006H |  | 00 | Lower byte of target address |
| 8007H |  | 85 | Higher byte of target address |
| 8008H | HLT | 76 | Halt execution |

Experiment 3: 16-Bit Addition

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | LXI H, 1234H | 21 | Load 1st 16-bit number into HL pair |
| 8001H |  | 34 | Lower byte of 1st number |
| 8002H |  | 12 | Higher byte of 1st number |
| 8003H | LXI D, 0002H | 11 | Load 2nd 16-bit number into DE pair |
| 8004H |  | 02 | Lower byte of 2nd number |
| 8005H |  | 00 | Higher byte of 2nd number |
| 8006H | DAD D | 19 | Double Add DE pair to HL pair (HL = HL + DE) |
| 8007H | SHLD 8500H | 22 | Store 16-bit result direct to 8500H & 8501H |
| 8008H |  | 00 | Lower byte of target address |
| 8009H |  | 85 | Higher byte of target address |
| 800AH | HLT | 76 | Halt execution |

Experiment 4: 16-Bit Subtraction

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | LXI H, 4567H | 21 | Load minuend into HL pair |
| 8001H |  | 67 | Lower byte of minuend |
| 8002H |  | 45 | Higher byte of minuend |
| 8003H | LXI B, 1234H | 01 | Load subtrahend into BC pair |
| 8004H |  | 34 | Lower byte of subtrahend |
| 8005H |  | 12 | Higher byte of subtrahend |
| 8006H | MOV A, L | 7B | Move lower byte of HL to Accumulator |
| 8007H | SUB C | 91 | Subtract lower byte of BC from Accumulator |
| 8008H | MOV L, A | 6F | Save result back in lower byte L |
| 8009H | MOV A, H | 7C | Move higher byte of HL to Accumulator |
| 800AH | SBB B | 98 | Subtract higher byte with Borrow from BC |
| 800BH | MOV H, A | 67 | Save result back in higher byte H |
| 800CH | SHLD 8500H | 22 | Store 16-bit result direct to 8500H & 8501H |
| 800DH |  | 00 | Lower byte of target address |
| 800EH |  | 85 | Higher byte of target address |
| 800FH | HLT | 76 | Halt execution |

Experiment 5: 8-Bit Multiplication (Repeated Addition)

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | MVI A, 05H | 3E | Load multiplicand into Accumulator |
| 8001H |  | 05 | Multiplicand value |
| 8002H | MOV E, A | 5F | Store multiplicand in Register E |
| 8003H | MVI B, 03H | 06 | Load multiplier into Register B (Loop Counter) |
| 8004H |  | 03 | Multiplier value |
| 8005H | MVI A, 00H | 3E | Clear Accumulator to hold running sum |
| 8006H |  | 00 | Initial sum value |
| 8007H | MVI C, 00H | 0E | Clear Register C to hold final overflow/carry |
| 8008H |  | 00 | Initial carry value |
| 8009H | ADD E | 83 | [LOOP] Add multiplicand to Accumulator |
| 800AH | JNC 800EH | D2 | Jump if No Carry flag set to address 800EH |
| 800BH |  | 0E | Lower byte of target loop address |
| 800CH |  | 80 | Higher byte of target loop address |
| 800DH | INR C | 0C | Increment Carry tracker if addition overflowed |
| 800EH | DCR B | 05 | [NEXT] Decrement the loop counter multiplier |
| 800FH | JNZ 8009H | C2 | Jump if Counter Not Zero back to loop (8009H) |
| 8010H |  | 09 | Lower byte of loop start address |
| 8011H |  | 80 | Higher byte of loop start address |
| 8012H | STA 8500H | 32 | Store lower byte of result at 8500H |
| 8013H |  | 00 | Lower byte of address |
| 8014H |  | 85 | Higher byte of address |
| 8015H | MOV A, C | 79 | Move Carry tracker to Accumulator |
| 8016H | STA 8501H | 32 | Store higher byte of result at 8501H |
| 8017H |  | 01 | Lower byte of address |
| 8018H |  | 85 | Higher byte of address |
| 8019H | HLT | 76 | Halt execution |

Experiment 6: 8-Bit Division (Repeated Subtraction)

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | MVI A, 0BH | 3E | Load dividend into Accumulator |
| 8001H |  | 0B | Dividend value |
| 8002H | MVI B, 03H | 06 | Load divisor into Register B |
| 8003H |  | 03 | Divisor value |
| 8004H | MVI C, 00H | 0E | Clear Register C to act as Quotient counter |
| 8005H |  | 00 | Initial quotient value |
| 8006H | CMP B | B8 | [LOOP] Compare Accumulator (dividend) with B (divisor) |
| 8007H | JC 800FH | DA | Jump if Carry (A < B) to end of loop at 800FH |
| 8008H |  | 0F | Lower byte of target address |
| 8009H |  | 80 | Higher byte of target address |
| 800AH | SUB B | 90 | Subtract divisor from current value inside Accumulator |
| 800BH | INR C | 0C | Increment the Quotient counter |
| 800CH | JMP 8006H | C3 | Jump unconditionally back to start of Compare loop |
| 800DH |  | 06 | Lower byte of loop start address |
| 800EH |  | 80 | Higher byte of loop start address |
| 800FH | STA 8501H | 32 | [END] Store remaining value (Remainder) at 8501H |
| 8010H |  | 01 | Lower byte of target address |
| 8011H |  | 85 | Higher byte of target address |
| 8012H | MOV A, C | 79 | Move Quotient counter into Accumulator |
| 8013H | STA 8500H | 32 | Store Quotient value at 8500H |
| 8014H |  | 00 | Lower byte of target address |
| 8015H |  | 85 | Higher byte of target address |
| 8016H | HLT | 76 | Halt execution |

Experiment 7: Addition of 10 Hex Numbers from Array

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | LXI H, 8200H | 21 | Load starting pointer of the data array into HL |
| 8001H |  | 00 | Lower byte of data array start address |
| 8002H |  | 82 | Higher byte of data array start address |
| 8003H | MVI B, 0AH | 06 | Load loop counter with total array size (10 = 0AH) |
| 8004H |  | 0A | Immediate count data |
| 8005H | MVI A, 00H | 3E | Clear Accumulator to aggregate total |
| 8006H |  | 00 | Initial sum |
| 8007H | MVI C, 00H | 0E | Clear Register C to capture multi-byte sum overflow |
| 8008H |  | 00 | Initial carry total |
| 8009H | ADD M | 86 | [LOOP] Add value from memory pointed by HL to Acc |
| 800AH | JNC 800EH | D2 | Jump if No Carry flag to target address 800EH |
| 800BH |  | 0E | Lower byte of target jump address |
| 800CH |  | 80 | Higher byte of target jump address |
| 800DH | INR C | 0C | Increment Overflow/Carry register pair |
| 800EH | INX H | 23 | [NEXT] Increment HL pointer to view next item in array |
| 800FH | DCR B | 05 | Decrement remaining element count loop tracker |
| 8010H | JNZ 8009H | C2 | Jump if items left (Counter Not Zero) to 8009H |
| 8011H |  | 09 | Lower byte of loop address |
| 8012H |  | 80 | Higher byte of loop address |
| 8013H | STA 8500H | 32 | Store total low-order summation byte at 8500H |
| 8014H |  | 00 | Lower byte of target address |
| 8015H |  | 85 | Higher byte of target address |
| 8016H | MOV A, C | 79 | Move tracking carry registry value to Accumulator |
| 8017H | STA 8501H | 32 | Store total high-order carry byte at 8501H |
| 8018H |  | 01 | Lower byte of target address |
| 8019H |  | 85 | Higher byte of target address |
| 801AH | HLT | 76 | Halt execution |

Experiment 8: Find the Largest (Biggest) Number in an Array

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | LXI H, 8200H | 21 | Load starting address of data array into HL |
| 8001H |  | 00 | Lower byte of array address |
| 8002H |  | 82 | Higher byte of array address |
| 8003H | MVI B, 05H | 06 | Load array element count into B |
| 8004H |  | 05 | Immediate block size array count |
| 8005H | MOV A, M | 7E | Initialize current maximum with 1st element of array |
| 8006H | DCR B | 05 | Decrement element loop tracker |
| 8007H | INX H | 23 | [LOOP] Increment memory pointer to target next element |
| 8008H | CMP M | BE | Compare current maximum (Acc) with next data element |
| 8009H | JNC 800DH | D2 | Jump if No Carry (Acc >= Memory) to step 800DH |
| 800AH |  | 0D | Lower byte of target address |
| 800BH |  | 80 | Higher byte of target address |
| 800CH | MOV A, M | 7E | Update maximum since element in memory is larger |
| 800DH | DCR B | 05 | [NEXT] Decrement remaining tracking element count |
| 800EH | JNZ 8007H | C2 | Jump if Counter Not Zero back to loop (8007H) |
| 800FH |  | 07 | Lower byte of loop start address |
| 8010H |  | 80 | Higher byte of loop start address |
| 8011H | STA 8500H | 32 | Store the recorded absolute maximum byte at 8500H |
| 8012H |  | 00 | Lower byte of target address |
| 8013H |  | 85 | Higher byte of target address |
| 8014H | HLT | 76 | Halt execution |

Experiment 9: Find the Smallest Number in an Array

| Memory Address | Instruction | Opcode / Data | Comments |
| --- | --- | --- | --- |
| 8000H | LXI H, 8200H | 21 | Load starting address of data array into HL |
| 8001H |  | 00 | Lower byte of array address |
| 8002H |  | 82 | Higher byte of array address |
| 8003H | MVI B, 05H | 06 | Load array element count into B |
| 8004H |  | 05 | Immediate block size array count |
| 8005H | MOV A, M | 7E | Initialize current minimum with 1st element of array |
| 8006H | DCR B | 05 | Decrement element loop tracker |
| 8007H | INX H | 23 | [LOOP] Increment memory pointer to target next element |
| 8008H | CMP M | BE | Compare current minimum (Acc) with next data element |
| 8009H | JC 800DH | DA | Jump if Carry (Acc < Memory) to skip update at 800DH |
| 800AH |  | 0D | Lower byte of target address |
| 800BH |  | 80 | Higher byte of target address |
| 800CH | MOV A, M | 7E | Update minimum since element in memory is smaller |
| 800DH | DCR B | 05 | [NEXT] Decrement remaining tracking element count |
| 800EH | JNZ 8007H | C2 | Jump if Counter Not Zero back to loop (8007H) |
| 800FH |  | 07 | Lower byte of loop start address |
| 8010H |  | 80 | Higher byte of loop start address |
| 8011H | STA 8500H | 32 | Store the recorded absolute minimum byte at 8500H |
| 8012H |  | 00 | Lower byte of target address |
| 8013H |  | 85 | Higher byte of target address |
| 8014H | HLT | 76 | Halt execution |

Appendix: Lab Memory Mapping Quick-Reference Guide

| Experiment Name | Code Start Address | Input Data Address | Output Address | Target Registers / Flags |
| --- | --- | --- | --- | --- |
| 1. 8-Bit Addition | 8000H | Immediate Data | 8500H | Acc (A) |
| 2. 8-Bit Subtraction | 8000H | Immediate Data | 8500H | Acc (A) |
| 3. 16-Bit Addition | 8000H | Immediate Data | 8500H - 8501H | HL Pair |
| 4. 16-Bit Subtraction | 8000H | Immediate Data | 8500H - 8501H | HL Pair (A & Borrow) |
| 5. 8-Bit Multiplication | 8000H | Immediate Data | 8500H - 8501H | Acc (A) & Carry (C) |
| 6. 8-Bit Division | 8000H | Immediate Data | 8500H - 8501H | Quotient (8500H), Rem (8501H) |
| 7. Addition of 10 Numbers | 8000H | 8200H - 8209H | 8500H - 8501H | HL Array Pointer, Acc, C |
| 8. Find Biggest Number | 8000H | 8200H - 8204H | 8500H | Acc (Max value), Carry Flag |
| 9. Find Smallest Number | 8000H | 8200H - 8204H | 8500H | Acc (Min value), Carry Flag |